# Lane B Fault-Injection Sweep — Summary Report

CE903 Group Project — Pod A (ASTRA-sim) deliverable for Lane B (LSTM training data)

# 1. Original Requirement (verbatim, from teammate)

The Lane B LSTM script (lane_b_lstm.py) needs three CSV files per seeded run. Right now the harness only writes fault_events.csv and stall_events.csv with 2 columns each. Here is exactly what needs to change:

1. New file: step_metrics.csv per run

One row per training step during the simulation. Columns:
step, t_step_start, step_time, completion_ratio, node_throughput, node_id
Where:

step = iteration number (0, 1, 2, ...)
t_step_start = cycle timestamp when that step started
step_time = how many cycles that step took
completion_ratio = fraction of collectives that completed (1.0 in healthy steps, drops when a stall happens)
node_throughput = effective throughput of the node in that step (can be derived from Wall time / step count)
node_id = which NPU (0-3)

2. Add stall_type column to stall_events.csv

Currently it only has collective_instance_id, t_stall_detected. It needs a third column stall_type which is either 'soft' (k=1.5x expected time) or 'hard' (k=3x expected time).

3. Run it 10 times with seeds 42–51, Aggressive sweep (MTBE 25 node-hours)

Output each run's three files into:
runs/seed_42/fault_events.csv
runs/seed_42/stall_events.csv
runs/seed_42/step_metrics.csv
runs/seed_43/...

Can you add this to run_fault_injection.py and push the 10 seed folders to the repo? Once those files are there I can run lane_b_lstm.py immediately and train the model.

And can you please tell me how long one run takes on your machine?

# 2. What the Existing Codebase Could Actually Support

Before writing any code, the existing fault-injection harness (ce903/pod_a_pipeline/run_fault_injection.py and ce903/fault_injection_hook/run_c_injection.py) was inspected. Key findings:

Device.disable() in the C++ hook is permanent and one-way — there is no enable()/recovery method.

The simulation supports exactly one fault per process run; once a node is disabled the simulation stalls and never logs a finite Wall time (confirmed by an existing code comment).

generate_synthetic_trace.py already only ever encodes ONE training step per subprocess invocation (forward pass → backward pass → AllReduce → optimizer step). There is no in-process multi-step loop in this engine at all.

The physical Ring topology embeds a static route into each Chunk at creation time, and the logical AllReduce ring is built once at startup with no pause/resume state. True in-process ring reformation would require rewriting ASTRA-sim's chunk-routing and collective-algorithm internals — multi-week simulator engineering, not a small hook extension.

Conclusion: literal in-process "fault → recover → continue" cannot be built on top of this engine without a significant rewrite of the simulator core.

# 3. Design Actually Implemented

Real recovery the only way this engine supports it: each training step is its own ASTRA-sim subprocess call, orchestrated from a new Python script.

## New file: ce903/fault_injection_hook/run_seed_sweep.py

Reuses existing helpers from run_c_injection.py (apply_patch, rebuild, revert_patch, run_astra_sim, parse_jct, generate_traces, make_network_config, make_system_config) — no changes were made to Device.h/.cpp or Topology.h/.cpp.

Healthy step: runs ASTRA-sim normally → real measured Wall time → completion_ratio = 1.0.

Hard fault (k=3.0): injects a real disable() for that step (proves the C++ stall actually fires), then permanently drops to N-1 NPUs for every subsequent step in that seed — reusing the existing, already-proven N→N-1 mechanism. step_time = 3.0 × expected_step_time; completion_ratio = 1/3.

Soft fault (k=1.5): same real disable() injection for that one step only; since the next step is a fresh process, the node count automatically reverts — i.e. it "recovers" for free. step_time = 1.5 × expected_step_time; completion_ratio = 1/1.5.

expected_step_time is measured once from a clean 4-NPU baseline run (the analytical backend has no internal RNG, so this is deterministic and reused across all seeds/steps).

## Deviation from the literal MTBE spec (flagged and confirmed with requester)

With only 10 steps per run, a literal Poisson process at MTBE = 25 node-hours (4 NPUs) would need ~35x more cumulative node-hours of exposure than a 10-step run can ever accumulate. This was verified empirically: the first full run produced zero fault events across all 10 seeds. Per explicit decision, each seed is now guaranteed 1–2 faults directly (MTBE_NODE_HOURS and the 10-step count are kept as descriptive labels for the 'Aggressive' sweep profile, but no longer drive the occurrence probability).

# 4. Output Files Produced

Location: ce903/pod_a_pipeline/runs/seed_42 .. seed_51 (also copied to ce903/lane_b_data/ for easier discovery).

| File | Columns |
| --- | --- |
| fault_events.csv | collective_instance_id, t_fault (one row per fault event) |
| stall_events.csv | collective_instance_id, t_stall_detected, stall_type (soft/hard) |
| step_metrics.csv | step, t_step_start, step_time, completion_ratio, node_throughput, node_id (one row per step per active node) |

Example (seed 45 — 2 faults: a hard fault at step 6 on node 2, a soft fault at step 7 on node 0):

Steps 0–5: 4 NPUs, healthy, step_time ≈ 95,767,236 cycles, completion_ratio = 1.0

Step 6: hard fault on node 2 → step_time = 287,301,708 (3×), completion_ratio = 0.333, node 2's throughput = 0

Step 7: soft fault on node 0 (3 NPUs remain after the hard fault) → step_time = 143,650,854 (1.5×), completion_ratio = 0.667, node 0's throughput = 0 for that step only

Steps 8–9: 3 NPUs (permanent after the step-6 hard fault), healthy, step_time ≈ 98,680,744 cycles

# 5. Verification Performed

Smoke-tested a single real faulted subprocess run before committing to the full sweep — confirmed the process exits cleanly (no hang) and produces no Wall time line, as expected.

Smoke-tested the N-1 (post hard-fault) path independently.

Ran the full 10-seed × 10-step sweep; verified every seed's three CSVs are internally consistent (fault/stall row counts match, step_metrics row counts match the expected node-drop pattern, completion_ratio < 1 only on fault steps).

Reverted the C++ patch and rebuilt the original ASTRA-sim binary afterward — the codebase is back to its pre-sweep state except for the new script and the generated seed_* output folders.

# 6. Timing

Total wall-clock time for the full job was a few minutes, almost entirely spent on the two one-time C++ rebuild cycles (apply patch → rebuild, and revert → rebuild), each taking a few minutes. The actual simulation work — 10 seeds × 10 steps = 100 lightweight ASTRA-sim subprocess calls — completed in well under a minute.

# 7. Known Caveats

lane_b_lstm.py was not found anywhere in the repository, so the exact column names/units/dtypes are matched to the spec as given, but unverified against the real parser.

node_throughput uses a placeholder formula (nominal work units / step_time) — easy to swap for a different unit once the LSTM script's expectations are confirmed.

Changes were committed locally only (branch: sprint5), not pushed to any remote.

# 8. Per-Seed Comparison Table

Quick comparison of fault outcomes across all 10 seeds — useful for sanity-checking the dataset's fault diversity before training (mix of hard vs soft, single vs double faults, and which steps were affected).

| Seed | # Faults | Stall Types | Faulted Step(s) | Final NPUs | Min completion_ratio | Total Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| 42 | 1 | soft | 0 | 4 | 0.667 | 1,005,555,978 |
| 43 | 1 | soft | 4 | 4 | 0.667 | 1,005,555,978 |
| 44 | 2 | soft, hard | 8, 9 | 4 | 0.333 | 1,197,090,450 |
| 45 | 2 | hard, soft | 6, 7 | 3 | 0.333 | 1,202,917,466 |
| 46 | 1 | hard | 6 | 3 | 0.333 | 1,157,947,356 |
| 47 | 2 | soft, hard | 1, 6 | 3 | 0.333 | 1,205,830,974 |
| 48 | 2 | soft, soft | 2, 8 | 4 | 0.667 | 1,053,439,596 |
| 49 | 1 | soft | 5 | 4 | 0.667 | 1,005,555,978 |
| 50 | 2 | soft, soft | 4, 5 | 4 | 0.667 | 1,053,439,596 |
| 51 | 1 | soft | 8 | 4 | 0.667 | 1,005,555,978 |

Totals across the 10-seed sweep: 15 fault events (4 hard, 11 soft). 3 of 10 seeds ended with a permanently reduced NPU count due to a hard fault; the rest recovered to 4 NPUs by the final step.