From the UIUC/IBM GPU Resilience Paper

Overall System Stats (pp. 1–2, 5, 10):

- Total GPUs studied: 1,168 (A40, A100, H100)

- Study period: 2.5 years (January 2022 – May 2024)

- Total GPU errors recorded: 63,253

- Overall system MTBE: 67 node hours

- Overall GPU node availability: 99.5% = 7 minutes downtime per GPU per day

- Average time to service a failed node (MTTR): 0.3 hours

- Total node hours lost due to GPU downtime: 5,700

MTBE Values by Error Type (from Table 1, p.6 — simulation parameters):

| Error Type | XID Code | Category | Per-Node MTBE (hrs) | Job Failure % |
| --- | --- | --- | --- | --- |
| Uncontained Memory Error | XID 95 | Memory | 2.17 | 97.2% |
| MMU Error | XID 31 | Hardware | 4.48 | 58.7% |
| NVLink Error | XID 74 | Interconnect | 28.28 | 65.7% |
| GSP Error | XID 119 | Hardware | 39.55 | 100% |
| PMU SPI Error | XID 122 | Hardware | 659.93 | 96.6% |
| GPU Fallen Off Bus | XID 79 | Hardware | 2724.86 | ~100% |
| Row Remapping Failure | XID 64 | Memory | 2413.45 | 100% |
| Double Bit ECC Error | XID 48 | Memory | 2639.71 | 90% |

Error Propagation Chains (from Figures 5, 6, 7, pp. 7-8— critical for FR-G3):

- GSP Error → GPU inoperable with probability 0.99 (propagation time: 0.2 seconds)

- GSP Error → PMU SPI Error with probability 0.01

- PMU SPI Error → MMU Error with probability 0.82 (propagation time: 0.05 seconds)

- PMU SPI Error → another PMU SPI Error with probability 0.18

- MMU Error → GPU operable but job interrupted with probability 0.37

- MMU Error → GPU in error state with probability 0.62 → requires GPU reset

- NVLink Error → another NVLink Error with probability 0.66 (propagation time: 0.1 seconds)

- NVLink Error → spreads to neighbouring GPUs with probability 0.14

- NVLink Error → GPU in error state with probability 0.20

- 84% of NVLink errors stay on one GPU; 16% spread to multiple GPUs on same node

- 5% of NVLink errors affected 4+ GPUs on the same node

Recovery Times (pp. 7,10):

- GSP error recovery: full node reboot required — one real case took 23 hours

- Node reboot can take up to 48 hours in practice at Delta

- Expected node service time (drain + reboot): 0.3 hours average

- 91% of useful GPU computation lost comes from errors persisting beyond P95 duration

Overprovisioning Impact (pp. 2,11):

- 800 GPUs, 40-min recovery time, 1% hourly failure rate → need 20% extra GPUs (160 extra)

- 800 GPUs, 5-min recovery time → overprovisioning drops to 5% (4× reduction)

- Improving availability from 99.5% to 99.9% → overprovisioning drops from 20% to 5%

- Cost of overprovisioning: over $1 million per month

GPU Memory vs Hardware Resilience (p. 6):

- GPU memory MTBE: ~26,093 node hours combined

- GPU hardware MTBE: ~800 node hours combined

- GPU memory is 30× more resilient than GPU hardware

From the L4 Paper

Failure Timing (when failures happen) p.3, Fig. 3:

- 74.1% of failures occur during iterative training — the phase our simulation focuses on

- 21.3% occur during launch/initialisation

- 57.5% of all failures are training crashes

- 16.6% are abnormal behaviours (hanging, slowing down)

- 4.7% are other causes

Failure Root Causes (from Section 3.3, p. 4-5):

- Hardware faults = #1 cause (exact % withheld for privacy but confirmed as most common)

- User faults = #2 cause

- Framework faults = small proportion

- Platform faults = smallest proportion

Hardware Fault Sub-types (ranked by frequency from most to least common):

| Fault Type | Description | Cascade Behaviour |
| --- | --- | --- |
| Network fault | Most common hardware fault — NIC link down, high error counts on network ports | Can cause performance degradation, hangs, full crashes |
| Accelerator fault | ECC memory errors, power faults, compute failures | Can cascade to full GPU unavailability |
| Node fault | Server failures — mainboard damage, power leakage, disk errors | Node becomes inaccessible; detected via heartbeat loss |
| Storage fault | Remote storage failures — checkpoint load/save failures | Can cause failures at any training stage |

Diagnosis Data (Table 1, p. 6):

- Average diagnosis time: 34.7 hours

- 41.9% of failures take more than 24 hours to diagnose

- 89.9% of failures require training logs for diagnosis

- Average log size per failed job: 16.92 GB

- 53.9% diagnosable from logs alone

- 36.0% require hybrid (logs + other monitoring data)

- 10.1% cannot be diagnosed from logs at all

Notable Real-World Examples from L4 (p.1):

- BLOOM-176B training: each hardware failure = average 1.5 hours of lost training time + 5-10 hours of recovery

- Llama3-405B: trained for 54 days on 16,384 GPUs, encountered 466 failures total

Parametrisation Table

| Failure Type | Source | Frequency / MTBE | Cascades To | Job Failure Probability | Suggested Simulation Parameter |
| --- | --- | --- | --- | --- | --- |
| GSP Error | UIUC/IBM Table 1 | MTBE: 39.55 node hrs | GPU inoperable (99%), PMU SPI (1%) | 100% | Node becomes unavailable; inject full node reboot with 0.3–23 hr recovery |
| PMU SPI Error | UIUC/IBM Table 1 | MTBE: 659.93 node hrs | MMU Error (82%), repeat PMU (18%) | 96.6% | Trigger MMU cascade with 82% probability |
| MMU Error | UIUC/IBM Table 1 | MTBE: 4.48 node hrs | Job interrupt (37%) or GPU error state (62%) | 58.7% | Probabilistic job kill; 62% chance GPU needs reset |
| NVLink Error | UIUC/IBM Table 1 | MTBE: 28.28 node hrs | Spreads to other GPUs (16%), GPU error state (20%) | 65.7% | Model as probabilistic inter-GPU propagation; 16% multi-GPU impact |
| Uncontained ECC | UIUC/IBM Table 1 | MTBE: 2.17 node hrs | Isolated (no cascade) | 97.2% | High frequency, isolated failure; GPU reset required |
| Network Fault | L4 Section 3.3 | Most common hardware fault (no exact rate given) | Performance degradation, hangs, crashes | High (not quantified) | Model as link flap or NIC down event |
| Node Fault | L4 Section 3.3 | Detected via heartbeat loss | Job crash on all GPUs on that node | Very high | Simulate heartbeat timeout → node removal |
| Storage Fault | L4 Section 3.3 | Can happen at any training stage | Checkpoint failure, job crash | Varies by stage | Simulate as checkpoint save/load failure event |

Notes:

- Network fault rate: L4 says network faults are the most common hardware fault but gives no specific MTBE or percentage — we may need to assume a rate or run a sensitivity test

- Node fault rate: Similarly described qualitatively in L4 but not quantified — heartbeat timeout threshold needs to be decided by us

- Storage fault rate: No rate given — may be out of scope for our simulation since ASTRA-sim models compute and network, not storage

- Cascade timing at scale: The UIUC/IBM propagation times (e.g. 0.05 seconds) are from a 1,168-GPU system — at 1,024 GPUs in our simulation, these should be approximately similar but worth noting

- GSP frequency under ML workloads: The paper notes GSP errors correlate with demanding ML benchmarks — our simulation running LLM training may see higher GSP rates than the MTBE average suggests

The two papers provide complementary failure data. The UIUC/IBM paper gives precise MTBE values and propagation probabilities for hardware-level GPU errors, while L4 gives us the failure taxonomy and timing distribution from a production platform perspective. Together they allow us to parameterise realistic failure scenarios for Sprint 4. The most important finding for our simulation is that GSP errors have 100% job failure probability and cascade with 99% probability, making them the highest-priority failure type to model. The main gap is the absence of a specific rate for network-level faults, which will require a team decision on assumed parameters.